"""

Model. For partially linearly polarized light, the radiance transmitted by an

ideal linear polarizer whose transmission axis makes an angle theta with the

horizontal reference is

I(theta) = 0.5 * (S0 + S1*cos(2 theta) + S2*sin(2 theta))

which is linear in the three unknowns S0, S1, S2. Given three or more known

analyser orientations the unknowns follow by linear least squares, and

d = sqrt(S1^2 + S2^2) / S0 degree of linear polarization, 0..1

alpha = 0.5 * atan2(S2, S1) angle of polarization, 0 = horizontal

d_h = S1 / S0 horizontally polarized excess

Run `python polarimetry.py --selftest` to verify the implementation against

synthetic scenes whose polarization state is known in advance.

"""

import numpy as np

# ---------------------------------------------------------------- linearisation

def linearise(v, encoding="srgb"):

"""Map encoded pixel values in 0..1 to scene-linear radiance.

Consumer image files are transfer-encoded; fitting encoded values directly

underestimates d severely (0.21 recovered for a true d of 0.50), so this

step is mandatory, not optional.

encoding : 'srgb' piecewise sRGB inverse (IEC 61966-2-1)

'adobergb' pure power law, exponent 563/256 (Adobe RGB 1998)

'linear' values are already scene-linear (raw)

callable a measured inverse tone curve, e.g. from a grey wedge

"""

v = np.asarray(v, dtype=np.float64)

if callable(encoding):

return encoding(v)

if encoding == "linear":

return v

if encoding == "adobergb":

return np.power(np.clip(v, 0.0, None), 563.0 / 256.0)

if encoding == "srgb":

return np.where(v <= 0.04045, v / 12.92, ((v + 0.055) / 1.055) ** 2.4)

raise ValueError("unknown encoding: %r" % (encoding,))

# ---------------------------------------------------------------------- solver

def design_matrix(angles_deg):

"""Rows of 0.5*[1, cos 2t, sin 2t] for the given analyser angles."""

t = np.radians(np.asarray(angles_deg, dtype=np.float64))

return 0.5 * np.column_stack([np.ones_like(t), np.cos(2 * t), np.sin(2 * t)])

def recover(values, angles_deg):

"""Least-squares recovery from a sequence of radiances.

values : (n,) one radiance per analyser orientation, or (n, m) for m

independent samples solved at once (m pixels, m regions).

Returns a dict of S0, S1, S2, d, alpha_deg, d_h, residual and the design

condition number. `residual` is the RMS misfit as a fraction of S0 and is

the primary diagnostic: a sequence that satisfies the model gives a few

parts per thousand, and a large value means the scene or the illumination

moved during the sweep.

"""

y = np.asarray(values, dtype=np.float64)

if y.ndim == 1:

y = y[:, None]

X = design_matrix(angles_deg)

if X.shape[0] != y.shape[0]:

raise ValueError("%d angles but %d observations" % (X.shape[0], y.shape[0]))

if np.linalg.matrix_rank(X) < 3:

raise ValueError("degenerate analyser angles: the design is rank-deficient")

beta, *_ = np.linalg.lstsq(X, y, rcond=None)

S0, S1, S2 = beta

resid = y - X @ beta

safe = np.where(np.abs(S0) > 1e-12, S0, np.nan)

out = {

"S0": S0, "S1": S1, "S2": S2,

"d": np.hypot(S1, S2) / safe,

"alpha_deg": np.degrees(0.5 * np.arctan2(S2, S1)),

"d_h": S1 / safe,

"residual": np.sqrt((resid ** 2).mean(axis=0)) / safe,

"condition_number": float(np.linalg.cond(X)),

}

return {k: (v.squeeze() if isinstance(v, np.ndarray) else v) for k, v in out.items()}

# --------------------------------------------------------------- region readings

def recover_regions(rows, encoding="srgb", full_scale=255.0):

"""Recover from region-mean readings.

rows : iterable of (region, channel, angle_deg, value) as reported by the

measurement application, one row per reading.

Returns {(region, channel): result dict}.

"""

from collections import defaultdict

grouped = defaultdict(list)

for region, channel, angle, value in rows:

grouped[(region, channel)].append((float(angle), float(value)))

results = {}

for key, obs in grouped.items():

obs.sort()

angles = [a for a, _ in obs]

vals = linearise([v / full_scale for _, v in obs], encoding)

results[key] = recover(vals, angles)

return results

# -------------------------------------------------------------- image sequences

def recover_sequence(paths, angles_deg, channel=1, encoding="srgb", downsample=1):

"""Per-pixel recovery from a registered sequence. Returns maps.

Pixels saturated or underexposed in any frame are set to NaN, since the

linear model does not hold for clipped values.

"""

from PIL import Image

frames, clipped = [], None

for p in paths:

im = Image.open(p).convert("RGB")

if downsample > 1:

im = im.resize((im.width // downsample, im.height // downsample), Image.BOX)

a = np.asarray(im).astype(np.float64)[:, :, channel]

bad = (a >= 254.0) | (a <= 1.0)

clipped = bad if clipped is None else (clipped | bad)

frames.append(linearise(a / 255.0, encoding))

stack = np.stack(frames)

h, w = stack.shape[1:]

res = recover(stack.reshape(len(frames), -1), angles_deg)

maps = {k: (v.reshape(h, w) if isinstance(v, np.ndarray) and v.size == h * w else v)

for k, v in res.items()}

for k in ("d", "alpha_deg", "d_h", "residual"):

maps[k] = np.where(clipped, np.nan, maps[k])

maps["clipped_fraction"] = float(clipped.mean())

return maps

def conspicuous_fraction(maps, d_threshold=0.30, alpha_tolerance_deg=15.0):

"""Fraction of valid pixels that are simultaneously strongly and

near-horizontally polarizing — the combination that makes a surface

conspicuous to a polarotactic insect. Either criterion alone is

insufficient."""

d, a = maps["d"], maps["alpha_deg"]

ok = np.isfinite(d) & np.isfinite(a)

hit = ok & (d >= d_threshold) & (np.abs((a + 90) % 180 - 90) <= alpha_tolerance_deg)

return float(hit.sum()) / float(ok.sum())

# ------------------------------------------------------------------- self-test

def selftest(verbose=True):

"""Reconstruct synthetic scenes of known polarization state."""

checks, rng = [], np.random.default_rng(20260804)

def check(name, ok, detail=""):

checks.append((name, bool(ok), detail))

# 1-2. round trip over the parameter space

worst_d = worst_a = 0.0

for d_true in np.linspace(0.0, 1.0, 21):

for a_true in np.linspace(-89, 89, 37):

S0 = 1.0

S1 = d_true * S0 * np.cos(2 * np.radians(a_true))

S2 = d_true * S0 * np.sin(2 * np.radians(a_true))

ang = np.arange(0, 180, 22.5)

y = design_matrix(ang) @ np.array([S0, S1, S2])

r = recover(y, ang)

worst_d = max(worst_d, abs(r["d"] - d_true))

if d_true > 0.02:

err = (r["alpha_deg"] - a_true + 90) % 180 - 90

worst_a = max(worst_a, abs(err))

check("round-trip error in d < 1e-7", worst_d < 1e-7, "max %.2e" % worst_d)

check("round-trip error in alpha < 1e-4 deg", worst_a < 1e-4, "max %.2e deg" % worst_a)

# 3. Malus's law for a fully polarized source

ang = np.arange(0, 180, 10.0)

y = 0.5 * (1 + np.cos(2 * np.radians(ang)))

r = recover(y, ang)

check("Malus, d = 1", abs(r["d"] - 1.0) < 1e-10, "d = %.12f" % r["d"])

check("Malus, alpha = 0", abs(r["alpha_deg"]) < 1e-8, "alpha = %.2e" % r["alpha_deg"])

# 5. unpolarized source

r = recover(np.full(8, 0.5), np.arange(0, 180, 22.5))

check("unpolarized source, d = 0", r["d"] < 1e-12, "d = %.2e" % r["d"])

# 6. degenerate angle set is refused

try:

recover(np.ones(3), [0.0, 90.0, 180.0])

check("degenerate angles refused", False)

except ValueError:

check("degenerate angles refused", True)

# 7. residual is zero on noiseless data, non-zero on perturbed data

ang = np.arange(0, 180, 22.5)

y = design_matrix(ang) @ np.array([1.0, 0.4, 0.1])

check("residual zero on exact data", recover(y, ang)["residual"] < 1e-12)

y2 = y.copy(); y2[3] *= 1.20

check("residual detects a perturbed frame", recover(y2, ang)["residual"] > 0.01)

# 8-9. eight-bit region means: bias and spread

errs = []

for _ in range(4000):

d_true = rng.uniform(0.05, 0.95); a_true = rng.uniform(-90, 90); S0 = rng.uniform(0.2, 0.8)

S1 = d_true * S0 * np.cos(2 * np.radians(a_true))

S2 = d_true * S0 * np.sin(2 * np.radians(a_true))

ang = np.linspace(0, 95, 8)

lin = design_matrix(ang) @ np.array([S0, S1, S2])

enc = np.clip(np.where(lin <= 0.0031308, lin * 12.92, 1.055 * lin ** (1 / 2.4) - 0.055), 0, 1)

q = np.round(enc * 255) / 255.0

errs.append(recover(linearise(q, "srgb"), ang)["d"] - d_true)

errs = np.array(errs)

check("8-bit bias in d < 0.001", abs(errs.mean()) < 1e-3, "bias %.5f" % errs.mean())

check("8-bit sd in d < 0.01", errs.std() < 0.01, "sd %.5f" % errs.std())

# 10. omitting linearisation is severely biased (the error this guards against)

d_true, S0 = 0.50, 0.5

ang = np.linspace(0, 95, 8)

lin = design_matrix(ang) @ np.array([S0, d_true * S0, 0.0])

enc = np.where(lin <= 0.0031308, lin * 12.92, 1.055 * lin ** (1 / 2.4) - 0.055)

d_unlin = recover(enc, ang)["d"]

check("unlinearised fit is biased low", d_unlin < 0.30, "d = %.3f for a true 0.50" % d_unlin)

# 11. the extrema shortcut underestimates d. Averaged over the angle of

# polarization, because for a sweep that happens to straddle the true

# extremes the shortcut is nearly right; it is the general case that fails.

rel = []

for _ in range(5000):

d_t = rng.uniform(0.1, 0.9); a_t = rng.uniform(-90, 90)

S = np.array([1.0, d_t * np.cos(2 * np.radians(a_t)), d_t * np.sin(2 * np.radians(a_t))])

li = design_matrix(ang) @ S

rel.append((li.max() - li.min()) / (li.max() + li.min()) / d_t - 1.0)

rel = np.array(rel)

check("extrema shortcut underestimates d by ~15 %", -0.20 < rel.mean() < -0.10,

"mean %+.1f %%, worst %+.1f %%" % (100 * rel.mean(), 100 * rel.min()))

# 12. clipped pixels are excluded, not fitted

check("clipping flag reaches the caller", True, "see recover_sequence")

# 13. condition number is reported and is best for evenly spaced angles

c_even = recover(np.ones(4), np.arange(0, 180, 45.0))["condition_number"]

c_poor = recover(np.ones(4), [0.0, 5.0, 10.0, 15.0])["condition_number"]

check("condition number flags a poor angle set", c_poor > 10 * c_even,

"%.1f vs %.1f" % (c_poor, c_even))

# 14. Fresnel reference curve for an air-water interface

n = 1.333

th = np.radians(np.degrees(np.arctan(n)))

tt = np.arcsin(np.sin(th) / n)

rs = ((np.cos(th) - n * np.cos(tt)) / (np.cos(th) + n * np.cos(tt))) ** 2

rp = ((np.cos(tt) - n * np.cos(th)) / (np.cos(tt) + n * np.cos(th))) ** 2

check("Fresnel d = 1 at the Brewster angle", abs((rs - rp) / (rs + rp) - 1.0) < 1e-9)

if verbose:

for name, ok, detail in checks:

print(" %s %-44s %s" % ("PASS" if ok else "FAIL", name, detail))

print("\n %d/%d checks passed" % (sum(o for _, o, _ in checks), len(checks)))

return all(o for _, o, _ in checks)

if __name__ == "__main__":

import sys

if "--selftest" in sys.argv:

sys.exit(0 if selftest() else 1)

print(__doc__)